Specifications

| Software: | bOS PRO Comfort License, Included: 3 Users |
| --- | --- |
| Size: | 107 mm x 61 mm x 50 mm (WxHxD) |
| CPU | 1,2 GHz quad-core ARM |
| RAM | 1 GB |
| Disk: | SD card 32 GB |
| Connectors: | 1 x KNX, 4 x USB, 1 x Ethernet port |
| Power: | 110-240VAC 0.35A 50/60 Hz |
| OS: | Linux |
| Power consumption: | approx. 2 W |
| Product # | CC-J-1 |

Please check our support tools for more information on how to work with bOS: www.comfortclick.com/Support

|  | - Always shut down Jigsaw properly by shutting it down in bOS Configurator. Don't shut down Jigsaw by unplugging it from the power supply. - The use of UPS is recommended. |
| --- | --- |

Jigsaw Quick Manual

- Installation - mount your Jigsaw on the DIN rail, connect the power supply, KNX and connect the Jigsaw to the network.

- Download and install bOS Configurator on your PC. You can get bOS Configurator on our web page: www.comfortclick.com/Software/Details/BOSConfig

- Connect to the Jigsaw

- Make sure that the PC with bOS Configurator is connected to the same network as your Jigsaw or Jigsaw is connected to the Internet and ports XXXX

- Open bOS Configurator

- Right click on the connection profile, then select Edit

- Enter your Access ID, User name and Password

Enter your Access ID. By default, your Access ID is the serial number of your Jigsaw (without - ; e.g. J1123). You can see the serial number on the sticker on the bottom of your Jigsaw.

Access ID is a unique name for accessing the bOS Server. This name enables adding profile and accessing your bOS Server, without the need of knowing it's IP address.

Enter Username: User and Password: Pass and click OK

- Edit Jigsaw Access Settings

- If you like, you can change your access ID and other access settings.

- Open bOS Configurator

- Click Edit button

- Login to your account on our web page

- You can now change your Server name, Access ID, Public Address and Local Address

- Click Update when you’re done.

- Start creating your bOS configuration or open an existing configuration.